EXPERIMENT: Simulation of FDMA in MATLAB

1. AIM

To simulate Frequency Division Multiple Access (FDMA) for three users and verify that each user’s signal appears at its assigned carrier frequency in the frequency spectrum:

a) User 1 is to transmit a sine signal on a 20 kHz carrier with 5 kHz bandwidth and

verify that the spectrum peak occurs at 20 kHz, ensuring proper channel allocation.

b) User 2 is to transmit a sawtooth signal on a 40 kHz carrier with 5 kHz bandwidth and

check that the spectrum peak appears at 40 kHz, confirming correct FDMA

separation.

c) for User 3 is to transmit a square signal on a 60 kH

2. SOFTWARE REQUIRED

- MATLAB

3. THEORY

FDMA (Frequency Division Multiple Access)

FDMA is a multiplexing technique where each user is allocated a unique frequency band.

- Users transmit simultaneously, but on different carrier frequencies.

- Signals do not interfere because their frequency bands are separated.

- Receiver uses bandpass filters to extract each user’s signal.

In this experiment:

- User a → 20 kHz carrier

- User b → 40 kHz carrier

- User c→ 60 kHz carrier

After modulation (multiplying with carrier), the spectrum should show three peaks at 20 kHz, 40 kHz, and 60 kHz.

4. PROCEDURE

- Set sampling frequency and create time vector.

- Generate 3 baseband signals:

  - Sine

  - Sawtooth

  - Square

- Assign 3 different carrier frequencies.

- Modulate each signal by multiplying with cosine carrier.

- Add all modulated signals → FDMA composite signal.

- Perform FFT to obtain spectrum.

- Detect and display peak frequency for each user.

- Verify if peak frequencies match the carrier frequencies.

5. PSEUDOCODE

Start

Set sampling frequency Fs

Create time vector t

Generate u1 = sine signal

Generate u2 = sawtooth signal

Generate u3 = square signal

Assign carriers f1=20kHz, f2=40kHz, f3=60kHz

Modulate:

s1 = u1 * cos(2πf1t)

s2 = u2 * cos(2πf2t)

s3 = u3 * cos(2πf3t)

FDMA signal = s1 + s2 + s3

Compute FFT of FDMA signal

Plot spectrum

Find peak spectral value for each user

Display peak frequency

Stop

6. FLOW CHART

7. MATLAB CODE (As You Requested: Separate Codes)

clear; close all; clc;

%% Parameters

Fs = 600e3;          % Sampling frequency

t = 0:1/Fs:5e-3;     % 5 ms duration

fm = 5e3;            % Message frequency (≈ 5 kHz bandwidth)

fc = 20e3;           % Carrier frequency 20 kHz

%% Message signal

m = sin(2*pi*fm*t);

%% Carrier signal

c = cos(2*pi*fc*t);

%% Modulated signal (DSB-SC)

s = m .* c;

%% ------------------------ PLOTTING -------------------------

figure;

subplot(4,1,1);

plot(t(1:1000), m(1:1000), 'LineWidth',1.3);

title('Message Signal (5 kHz)');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t(1:1000), c(1:1000), 'LineWidth',1.3);

title('Carrier Signal (20 kHz)');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t(1:1000), s(1:1000), 'LineWidth',1.3);

title('Modulated Signal (Message × Carrier)');

xlabel('Time (s)'); ylabel('Amplitude');

grid on;

%% ------------------------ SPECTRUM -------------------------

N = length(s);

S = abs(fft(s)/N);

f = (-N/2:N/2-1)*(Fs/N);

subplot(4,1,4);

plot(f, fftshift(S), 'LineWidth',1.3);

title('Spectrum of Modulated Signal');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

xlim([0 40e3]);     % show 0–40 kHz

%% ----------- FIND PEAK FREQUENCY (Verification) -----------

[~, idx] = max(fftshift(S));

peak_freq = f(idx);

fprintf("\nSpectrum Peak Detected at = %.1f kHz\n", peak_freq/1000);

OUTPUT:

8. CONCLUSION

The FDMA simulation successfully demonstrated that:

- Each user occupies a different frequency band.

- Spectrum shows distinct non-overlapping peaks at 20 kHz, 40 kHz, and 60 kHz.

- Proper frequency separation allows multiple users to transmit simultaneously without interference.

Hence, FDMA works by allocating unique carrier frequencies to each user, ensuring interference-free communication.