10.NB-IoT vs LTE-M Throughput Simulation

Aim

To simulate and compare the throughput of NB-IoT and LTE-M based on their bandwidth and modulation schemes.

Software Required

MATLAB / Octave

Basic plotting functions only

Theory (Simple)

NB-IoT uses 180 kHz bandwidth and usually QPSK modulation.

LTE-M uses 1.4 MHz bandwidth and supports QPSK / 16QAM.

Throughput ≈ Bandwidth × log₂(M) × (1 – BER)

Higher bandwidth + higher modulation = higher throughput.

Procedure (Simple)

Define bandwidth for NB-IoT and LTE-M.

Choose modulations (NB-IoT = QPSK, LTE-M = QPSK & 16QAM).

Sweep SNR from –5 to 25 dB.

Compute BER using simple function.

Compute throughput.

Plot and compare both systems.

Pseudocode (Simple)

set BW_NB = 180 kHz

set BW_LTEM = 1.4 MHz

mod_NB = QPSK (M=4)

mod_LTEM = 16QAM (M=16)

for SNR = -5 to 25 dB

compute BER for each modulation

compute throughput = BW * log2(M) * (1-BER)

plot NB-IoT and LTE-M throughput vs SNR

Code

clear; clc; close all;

%% PARAMETERS

BW = [180e3, 1.4e6];           % Bandwidth: NB-IoT, LTE-M (Hz)

latency = [1, 0.1];             % Latency in seconds (NB-IoT, LTE-M)

SNR_dB = 0:5:30;                % SNR range in dB

tech = {'NB-IoT','LTE-M'};      % Technology names

%% Throughput vs SNR

throughput = zeros(length(BW), length(SNR_dB));

for t = 1:length(BW)

for s = 1:length(SNR_dB)

snr_linear = 10^(SNR_dB(s)/10);

C = BW(t) * log2(1 + snr_linear);          % Shannon capacity

throughput(t,s) = C * (1/latency(t));      % Adjust for latency

end

end

% Plot Throughput vs SNR

figure; hold on;

colors = {'b','r'};

for t = 1:length(BW)

plot(SNR_dB, throughput(t,:)/1e3, 'LineWidth',2,'Color',colors{t}); % kbps

text(SNR_dB(end)+0.5, throughput(t,end)/1e3, tech{t}, 'Color',colors{t}, ...

'FontWeight','bold','FontSize',10);

end

grid on;

xlabel('SNR (dB)');

ylabel('Throughput (kbps)');

title('NB-IoT vs LTE-M Throughput vs SNR');

%% Throughput vs Bandwidth at Fixed SNR

fixed_SNR_dB = 20;  % Example SNR

throughput_bw = zeros(length(BW),1);

for t = 1:length(BW)

snr_linear = 10^(fixed_SNR_dB/10);

C = BW(t) * log2(1 + snr_linear);

throughput_bw(t) = C * (1/latency(t));

end

% Plot Throughput vs Bandwidth

figure;

bar(throughput_bw/1e3);  % kbps

set(gca,'XTickLabel',tech,'FontWeight','bold');

ylabel('Throughput (kbps)');

title(['Throughput vs Bandwidth at SNR = ' num2str(fixed_SNR_dB) ' dB']);

grid on;

OUTPUT:

Result (Very Simple)

LTE-M provides much higher throughput than NB-IoT at all SNR levels because it uses larger bandwidth and higher-order modulation.