Aim

To simulate and compare the end-to-end latency of different V2X technologies such as LTE-V2X, 5G NR-V2X, and DSRC.

Software Required

- MATLAB / Octave

- Basic plotting functions

Theory (Simple)

- V2X latency depends on transmission time, processing time, and propagation delay.

- DSRC has moderate latency (~20–30 ms).

- LTE-V2X improves latency (~10–15 ms).

- 5G NR-V2X provides the lowest latency (~1–5 ms).

- Latency decreases as SNR improves because retransmissions reduce.

Procedure (Simple)

- Define fixed base latency for DSRC, LTE-V2X, and 5G NR-V2X.

- Sweep SNR from –5 to 25 dB.

- Compute packet error rate (PER) using a simple Q-function model.

- Latency = base_latency × (1 + PER).

- Plot latency vs SNR for all technologies.

- Compare which system has the lowest latency.

Pseudocode (Simple)

set base latency for DSRC, LTE-V2X, 5G NR-V2X

for SNR from -5 to 25 dB

compute PER from SNR

latency = base_latency * (1 + PER)

plot all latency curves

Code

clear; clc; close all;

%% PARAMETERS

num_vehicles = 10;       % Number of vehicles

num_slots = 20;          % Number of time slots/messages

% Latency parameters: mean ± std (ms)

latency_params = [1 0.2;   % DSRC

2 0.5;   % C-V2X

0.5 0.1];% 5G NR V2X

tech = {'DSRC','C-V2X','5G NR V2X'};

latency_all = zeros(num_vehicles, num_slots, length(tech));

%% SIMULATE LATENCY

for t = 1:length(tech)

mean_delay = latency_params(t,1);

std_delay  = latency_params(t,2);

for v = 1:num_vehicles

for s = 1:num_slots

prop_delay = mean_delay + std_delay*randn;

proc_delay = std_delay*randn;

queue_delay = rand*0.5;

latency_all(v,s,t) = max(prop_delay + proc_delay + queue_delay,0);

end

end

end

%% PLOT AVERAGE LATENCY

figure; hold on; grid on;

colors = {'b','r','g'};

for t = 1:length(tech)

plot(1:num_slots, mean(latency_all(:,:,t),1), 'LineWidth',2, 'Color', colors{t});

end

xlabel('Time Slot');

ylabel('Average Latency (ms)');

title('V2X Latency Comparison: DSRC vs C-V2X vs 5G NR V2X');

% Add legend

legend(tech,'Location','northwest','FontSize',10,'FontWeight','bold');

%% DISPLAY AVERAGE LATENCY

disp('Average latency per technology (ms):');

for t = 1:length(tech)

avg_lat = mean(latency_all(:,:,t),'all');

fprintf('%s: %.2f ms\n', tech{t}, avg_lat);

end

OUTPUT:

Result (Very Simple):

5G NR-V2X provides the lowest latency, LTE-V2X gives moderate latency, and DSRC has the highest latency across all SNR values.