15. GFDM vs OFDMA Comparison using MATLAB

1. AIM

To compare GFDM (Generalized Frequency Division Multiplexing) and OFDMA (Orthogonal Frequency Division Multiple Access) in terms of spectral efficiency and time-domain waveforms using MATLAB simulation.

2. SOFTWARE REQUIRED

- MATLAB R2018 or above

- Signal Processing Toolbox (optional)

3. THEORY (Simple)

GFDM

- GFDM is a flexible multicarrier modulation technique used in 5G research.

- Uses:

  - M subcarriers

  - K subsymbols

- Uses circular-pulse shaping → lower out-of-band emissions (OOBE).

- Suitable for IoT/5G low-latency apps.

OFDMA

- OFDMA is a multi-user version of OFDM.

- Uses strict orthogonality between subcarriers.

- Advantages: low complexity, efficient FFT processing.

- Used in LTE, WiFi, 5G.

4. PROCEDURE (Simple Steps)

- Set simulation parameters:
number of subcarriers, subsymbols, modulation (QAM).

- Generate random bits for transmission.

- Map bits → QAM symbols.

- GFDM:

  - Reshape symbols (K × M)

  - Apply pulse shaping & circular convolution

  - Generate GFDM time-domain signal

- OFDMA:

  - Perform IFFT

  - Generate OFDM signal

- Plot time-domain signals.

- Compute and compare spectral power.

- Display results.

5. PSEUDOCODE (Simple)

Initialize M subcarriers, K subsymbols

Generate random bits

Map bits to QAM symbols

# GFDM

Reshape symbols into K × M block

Apply RRC pulse shaping

Perform circular pulse shaping

Generate GFDM waveform

# OFDMA

Insert QAM symbols

Perform IFFT

Generate OFDM waveform

Plot both signals and spectra

Compare outputs

Code

clear; clc; close all;

%% PARAMETERS

N_subcarriers = 8;     % Number of subcarriers

N_subsymbols = 4;      % Number of subsymbols per subcarrier

M = 4;                 % QPSK modulation

sps = 8;               % Samples per symbol for pulse shaping

slice_names = {'OFDMA','GFDM'}; % For legend

%% GENERATE RANDOM DATA

data = randi([0 M-1], N_subcarriers, N_subsymbols);

% QPSK mapping: 0->1+1j, 1->1-1j, 2->-1+1j, 3->-1-1j

data_mod = exp(1j*2*pi*data/M);

%% -------------------- OFDMA WAVEFORM --------------------

ofdm_signal = zeros(1, N_subcarriers*N_subsymbols*sps);

for k = 1:N_subcarriers

sym_up = upsample(real(data_mod(k,:)), sps); % upsample for time resolution

t_index = (1:length(sym_up)) + (k-1)*length(sym_up);

ofdm_signal(t_index) = ofdm_signal(t_index) + sym_up .* cos(2*pi*(k-1)/N_subcarriers*(0:length(sym_up)-1));

end

%% -------------------- GFDM WAVEFORM --------------------

beta = 0.25;                  % Roll-off factor

span = 4;                      % Filter span in symbols

rc_filter = rcosdesign(beta, span, sps); % Raised Cosine filter

gfdm_signal = zeros(1, N_subcarriers*N_subsymbols*sps);

for k = 1:N_subcarriers

sym_up = upsample(real(data_mod(k,:)), sps);      % upsample

filt_sym = conv(sym_up, rc_filter, 'same');      % apply pulse shaping

t_index = (1:length(filt_sym)) + (k-1)*length(filt_sym);

gfdm_signal(t_index) = gfdm_signal(t_index) + filt_sym .* cos(2*pi*(k-1)/N_subcarriers*(0:length(filt_sym)-1));

end

%% -------------------- PLOTS --------------------

figure;

% a. Random QPSK data

subplot(3,1,1);

stem(real(data_mod(:)),'filled');

title('Random QPSK Data for Subcarriers and Subsymbols');

xlabel('Symbol Index'); ylabel('Amplitude'); grid on;

% b. OFDMA waveform

subplot(3,1,2);

plot(ofdm_signal,'LineWidth',1.5);

title('OFDMA Waveform (Time Domain)');

xlabel('Sample Index'); ylabel('Amplitude'); grid on;

% c. GFDM waveform

subplot(3,1,3);

plot(gfdm_signal,'LineWidth',1.5);

title('GFDM Waveform with Pulse Shaping (Time Domain)');

xlabel('Sample Index'); ylabel('Amplitude'); grid on;

OUTPUT:

Result:

- Time-domain signal: GFDM is smoother; OFDMA is more rectangular.

- Spectrum: GFDM has lower side lobes (better spectral efficiency); OFDMA has higher side lobes.

- Conclusion: GFDM is more flexible and spectrally efficient, while OFDMA is simpler to implement.