16.mmWave Loss Components (28 GHz & 60 GHz)

1. AIM

To calculate and compare mmWave path loss components at 28 GHz and 60 GHz using a basic free-space model in MATLAB.

2. SOFTWARE REQUIRED

- MATLAB (any version 2015+)

- No extra toolboxes needed

3. THEORY (Simple)

Why mmWave?

Millimeter-wave bands (28 GHz, 60 GHz) are used in 5G for high data rates but suffer from high propagation loss.

Key Loss Components

1. Free Space Path Loss (FSPL)

FSPL(dB)=32.44+20log⁡10(fMHz)+20log⁡10(dkm)

2. Atmospheric Absorption

- At 28 GHz → low absorption

- At 60 GHz → very high absorption due to oxygen peak (~15 dB/km)

3. Rain/Fog Loss (simplified)

- Increases with frequency

- Heavy rain: 10–20 dB/km at mmWave

4. PROCEDURE (Simple)

- Choose frequencies: 28 GHz, 60 GHz

- Choose a distance range: 1 m to 1000 m

- Convert distance to km and frequency to MHz

- Apply FSPL formula

- Add atmospheric absorption (constant per km)

- Plot both curves

- Compare losses

5. PSEUDOCODE

Set frequency1 = 28 GHz

Set frequency2 = 60 GHz

Create distance vector d = 1 to 1000 m

Convert distances to km

Compute FSPL for both frequencies

Add atmospheric absorption (0.5 dB/km for 28 GHz, 15 dB/km for 60 GHz)

Plot results

CODE

clear; clc; close all;

%% PARAMETERS

frequencies = [28e9 60e9];        % Frequencies in Hz

c = 3e8;                          % Speed of light (m/s)

distances = 1:1:100;              % Distance in meters

epsilon_r_wall = 5;               % Relative permittivity of wall

theta_inc = 45;                    % Incident angle in degrees

% Atmospheric attenuation (dB/m)

atm_loss = [0.06, 0.2];           % 28GHz, 60GHz

%% INITIALIZE ARRAYS

ReflectionLoss = zeros(length(frequencies), length(distances));

PenetrationLoss = zeros(length(frequencies), length(distances));

AtmosphericLoss = zeros(length(frequencies), length(distances));

%% COMPUTE LOS / LOS COMPONENTS

for f_idx = 1:length(frequencies)

f = frequencies(f_idx);

for d_idx = 1:length(distances)

d = distances(d_idx);

% -------- Reflection Loss (Fresnel coefficient, perpendicular polarization) --------

theta_rad = deg2rad(theta_inc);

gamma = (cos(theta_rad) - sqrt(epsilon_r_wall - sin(theta_rad)^2)) / ...

(cos(theta_rad) + sqrt(epsilon_r_wall - sin(theta_rad)^2));

ReflectionLoss(f_idx,d_idx) = abs(gamma)^2;             % Power reflection coefficient

ReflectionLoss(f_idx,d_idx) = -10*log10(ReflectionLoss(f_idx,d_idx)+eps); % dB

% -------- Penetration Loss (frequency-dependent) --------

PenetrationLoss(f_idx,d_idx) = 5 + 0.02*f/1e9;         % Example: 28GHz ~5.56dB, 60GHz ~6.2dB

% -------- Atmospheric Loss (linear with distance) --------

AtmosphericLoss(f_idx,d_idx) = atm_loss(f_idx) * d;     % dB

end

end

%% -------------------- PLOTS --------------------

figure;

% Reflection Loss

plot(distances, ReflectionLoss(1,:),'r','LineWidth',1.5); hold on;

plot(distances, ReflectionLoss(2,:),'b','LineWidth',1.5);

% Penetration Loss

plot(distances, PenetrationLoss(1,:),'r--','LineWidth',1.5);

plot(distances, PenetrationLoss(2,:),'b--','LineWidth',1.5);

% Atmospheric Loss

plot(distances, AtmosphericLoss(1,:),'r-.','LineWidth',1.5);

plot(distances, AtmosphericLoss(2,:),'b-.','LineWidth',1.5);

grid on;

xlabel('Distance (m)'); ylabel('Loss (dB)');

title('mmWave Loss Components at 28 GHz and 60 GHz');

legend('Reflection 28 GHz','Reflection 60 GHz', ...

'Penetration 28 GHz','Penetration 60 GHz', ...

'Atmospheric 28 GHz','Atmospheric 60 GHz');

OUTPUT:

RESULT:

- At the same distance, 60 GHz has much higher total loss than 28 GHz.

- This is due to higher free-space loss and strong oxygen absorption (~15 dB/km) at 60 GHz.

- Therefore, 28 GHz is suitable for longer-range 5G, while 60 GHz is better for short-range high-capacity links.