19.5G network slicing environment

1. AIM

To study and simulate a 5G Network Slicing Environment, showing how multiple virtual networks (slices) can operate over the same physical 5G infrastructure with different Quality-of-Service (QoS) requirements.

2. SOFTWARE REQUIRED

Any one of the following:

- MATLAB

- Python (NumPy + basic simulation)

- Optional: Mininet/NS-3 for advanced slicing simulation (not required for simple model)

3. THEORY (Simple)

What is Network Slicing?

- A 5G feature that allows the same physical network to be divided into multiple virtual networks, each optimized for different applications.

Types of 5G Slices

- eMBB – Enhanced Mobile Broadband

  - High data rate, high throughput

- URLLC – Ultra-Reliable Low-Latency Communication

  - <1 ms latency

- mMTC – Massive Machine Type Communication

  - Thousands/millions of IoT devices

Why Slicing?

- Different applications (VR, IoT, autonomous vehicles) need different performance.

- Slicing assigns bandwidth, latency, priority separately for each slice.

Basic Idea

One physical network → multiple isolated virtual networks
Each slice has:

- Its own bandwidth share

- Its own latency requirement

- Its own traffic priority

4. PROCEDURE (Simple)

- Define three slices: eMBB, URLLC, mMTC.

- Assign bandwidth, latency, priority values to each slice.

- Generate random traffic for each slice.

- Simulate scheduling:

  - URLLC → highest priority

  - eMBB → medium

  - mMTC → low

- Allocate resources based on priority and load.

- Display total throughput and average latency per slice.

5. PSEUDOCODE (Simple)

Define slices: eMBB, URLLC, mMTC

Assign bandwidth and latency targets

Generate traffic for each slice

For each time-step:

Sort slices by priority

Allocate bandwidth

Measure throughput and latency

Print final performance for each slice

CODE:

clear; clc; close all;

%% Total available bandwidth (MHz)

BW_total = 100;

%% Slice Requirements

% eMBB: High throughput

% URLLC: Ultra low latency, high priority

% mMTC: Large number of devices, small packets

req_eMBB = 50;   % Required Mbps

req_URLLC = 20;  % Required Mbps

req_mMTC = 10;   % Required Mbps

priority = [2 3 1];  % 1=low, 3=high priority (URLLC > eMBB > mMTC)

%% Dynamic Bandwidth Allocation

BW_alloc = zeros(1,3);

req = [req_eMBB req_URLLC req_mMTC];

% Weighted allocation based on priority

priority_weights = priority ./ sum(priority);

BW_alloc = BW_total * priority_weights;

%% Compute Throughput (T = BW * spectral efficiency)

SE_eMBB = 6;    % bits/s/Hz

SE_URLLC = 4;

SE_mMTC = 1;

th_eMBB = BW_alloc(1)*1e6 * SE_eMBB / 1e6;

th_URLLC = BW_alloc(2)*1e6 * SE_URLLC / 1e6;

th_mMTC = BW_alloc(3)*1e6 * SE_mMTC / 1e6;

%% Latency Simulation (URLLC minimum)

lat_eMBB = 20 + rand*10;    % ms

lat_URLLC = 1 + rand*0.5;   % ms

lat_mMTC = 50 + rand*20;    % ms

%% Print Results

fprintf("\n===== Network Slicing Simulation Results =====\n");

fprintf("Allocated Bandwidth (MHz): eMBB=%.2f, URLLC=%.2f, mMTC=%.2f\n", BW_alloc(1), BW_alloc(2), BW_alloc(3));

fprintf("Throughput (Mbps): eMBB=%.2f, URLLC=%.2f, mMTC=%.2f\n", th_eMBB, th_URLLC, th_mMTC);

fprintf("Latency (ms): eMBB=%.2f, URLLC=%.2f, mMTC=%.2f\n", lat_eMBB, lat_URLLC, lat_mMTC);

%% Plot Throughput Comparison

figure;

bar([th_eMBB, th_URLLC, th_mMTC]);

set(gca, 'XTickLabel', {'eMBB','URLLC','mMTC'});

ylabel('Throughput (Mbps)');

title('Slice-wise Throughput Performance');

%% Plot Latency Comparison

figure;

bar([lat_eMBB, lat_URLLC, lat_mMTC]);

set(gca, 'XTickLabel', {'eMBB','URLLC','mMTC'});

ylabel('Latency (ms)');

title('Slice-wise Latency Performance');

%% Plot Bandwidth Allocation

figure;

pie(BW_alloc, {'eMBB','URLLC','mMTC'});

title('Dynamic Bandwidth Allocation for 3 Network Slices');

OUTPUT:

RESULT:

5G network slicing allows different virtual networks (eMBB, URLLC, mMTC) to share the same physical 5G system while each slice gets its own optimized performance for speed, latency, or device density.