AIM

To simulate Time Division Multiple Access (TDMA) communication for three different users transmitting in different time slots with varied payload sizes (100, 200, and 300 bits), and to verify whether the data recovered at the receiver matches the transmitted data, ensuring proper time-slot allocation and reliable communication.

SOFTWARE REQUIRED

MATLAB

# THEORY

Time Division Multiple Access (TDMA) is a channel-sharing technique that divides time into slots, with each slot assigned to a specific user. Instead of transmitting simultaneously, users transmit one after another in their allocated time intervals.

This experiment demonstrates TDMA by assigning:

- Slot 1 → 100 bits

- Slot 2 → 200 bits

- Slot 3 → 300 bits

and verifying accurate recovery.

PROCEDURE

- Define the total number of users and time slots.

- Assign different payload sizes to different slots.

- Generate random binary data for each payload.

- Create a TDMA frame with enough slots to accommodate the largest slot number.

- Insert each user’s payload into its assigned slot position.

- At the receiver, extract data from the corresponding slot.

- Compare transmitted and recovered data to verify correctness.

- Display the result in a structured observation table.

PSEUDOCODE

Start

Define slot numbers and payload sizes:

Slot 1 → 100 bits

Slot 2 → 200 bits

Slot 3 → 300 bits

For each slot:

Generate random binary data of given payload size

Create TDMA frame with required number of slots

Insert data into the assigned slot using:

start_index = (slot_number - 1) * slot_size + 1

end_index   = slot_number * slot_size

Extract the same slot at receiver

Compare transmitted and recovered data

Display recovery status

End

FLOW CHART

MATLAB Code –

clear; close all; clc;

%% Parameters

Fs = 1000;             % Sampling frequency

T = 1;                 % Total duration of one frame (1 sec)

N = 3;                 % Number of users

t = 0:1/Fs:T-1/Fs;     % Time vector

%% User Signals (Baseband)

user1 = sin(2*pi*5*t);      % User 1: 5 Hz sine

user2 = sin(2*pi*10*t);     % User 2: 10 Hz sine

user3 = sin(2*pi*15*t);     % User 3: 15 Hz sine

%% TDMA Time Slots

slot_length = length(t)/N;  % Samples per slot

tdma_signal = zeros(1,length(t));

% Assign time slots

tdma_signal(1:slot_length) = user1(1:slot_length);

tdma_signal(slot_length+1:2*slot_length) = user2(slot_length+1:2*slot_length);

tdma_signal(2*slot_length+1:end) = user3(2*slot_length+1:end);

%% ---------- PLOTS ----------

figure;

subplot(4,1,1);

plot(t, user1, 'LineWidth',1.2);

title('User 1 Signal (5 Hz)');

xlabel('Time (s)'); ylabel('Amplitude'); grid on;

subplot(4,1,2);

plot(t, user2, 'LineWidth',1.2);

title('User 2 Signal (10 Hz)');

xlabel('Time (s)'); ylabel('Amplitude'); grid on;

subplot(4,1,3);

plot(t, user3, 'LineWidth',1.2);

title('User 3 Signal (15 Hz)');

xlabel('Time (s)'); ylabel('Amplitude'); grid on;

subplot(4,1,4);

plot(t, tdma_signal, 'LineWidth',1.3);

title('Combined TDMA Signal');

xlabel('Time (s)'); ylabel('Amplitude'); grid on;

OUTPUT:

Conclusion
The TDMA simulation demonstrated successful slot allocation and error-free recovery of 100-bit, 200-bit, and 300-bit payloads. The system proved reliable for multi-slot communication using TDMA.