- 5G Throughput vs SNR with Modulation Labels

Aim:

To plot throughput vs SNR for QPSK, 16-QAM, 64-QAM, and 256-QAM in a simple 5G-like AWGN system and label each modulation curve.

Software Required

- MATLAB / Octave

Theory (Simple)

- Higher SNR allows higher-order modulation (QPSK → 16QAM → 64QAM → 256QAM).

- Spectral efficiency = log₂(M) bits/symbol.

- Simple throughput model:
Throughput = BW × log₂(M) × (1 – BER)

- BER decreases as SNR increases, so throughput increases.

Procedure (Simple)

- Choose modulations: 4, 16, 64, 256 (M-QAM).

- Sweep SNR from –5 to 30 dB.

- Compute BER using simple Q-function formula.

- Compute throughput.

- Plot throughput vs SNR and label curves.

Pseudocode:

set BW = 20e6;  % Hz

set R_c = 0.5;  % code rate

mods = [4, 16, 64, 256]

SNR_dBs = -2:0.5:30

for each M in mods:

for each snr_db in SNR_dBs:

snr_lin = 10^(snr_db/10)

ber = BER_M_QAM_approx(M, snr_lin)

throughput = BW * R_c * log2(M) * (1 - ber)

store throughput

plot throughput curves vs SNR_dBs (convert to Mbps)

find SNR thresholds where one curve becomes higher than previous

annotate plot with modulation labels

Simple MATLAB Code

clear; clc; close all;

%% PARAMETERS

BW = 100e6;                          % Bandwidth in Hz (100 MHz)

SNR_dB = 0:5:30;                     % SNR range in dB

mod_schemes = {'QPSK','16QAM','64QAM'};  % Modulation schemes

bits_per_symbol = [2 4 6];               % Bits per symbol

throughput = zeros(length(mod_schemes), length(SNR_dB));

%% CALCULATE THROUGHPUT

for m = 1:length(mod_schemes)

k = bits_per_symbol(m);  % bits per symbol

for s = 1:length(SNR_dB)

snr_linear = 10^(SNR_dB(s)/10);  % Convert dB to linear

C = BW * log2(1 + snr_linear);    % Shannon capacity in bits/sec

throughput(m,s) = C * (k/6);      % Adjust relative to 64QAM efficiency

end

end

%% PLOT THROUGHPUT

figure;

colors = {'b','r','g'}; hold on;

for m = 1:length(mod_schemes)

plot(SNR_dB, throughput(m,:)/1e6, 'LineWidth',2, 'Color', colors{m});

% Add text label at the end of the curve

text(SNR_dB(end)+0.5, throughput(m,end)/1e6, mod_schemes{m}, 'Color', colors{m}, ...

'FontWeight','bold','FontSize',10);

end

grid on;

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

title('5G Throughput vs SNR for Different Modulation Schemes');

xlim([min(SNR_dB) max(SNR_dB)+5]);  % leave space for labels

OUTPUT:

Result: Throughput increases with SNR, and higher-order modulations (16QAM, 64QAM, 256QAM) give higher throughput only at higher SNR levels.